## Space Mission Control and Operations Division

# Incident Report

| Reported by | Keziah Bartilad |
| --- | --- |
| Position | Senior Science Research Specialist |
| Date of Report | 2026-01-13 |
| Incident No. | SMCOD-IR-GS-QC-2026-0001 |

# Incident Details

| Date (YYYY-MM-DD) | 2026-01-13 |
| --- | --- |
| Time | 05:51:31 |
| Location | QG GS |
| Current Status | Resolved |

# Incident Description

## Nature of Incident

The ACU logged repeated communication errors with the hardware at the RF cabinet and at the Antenna. After self-resetting a number of times, the issue with the connections resolved.

## Sequence of Events

| 12/01/2026 | 21:51:31 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| --- | --- | --- | --- |
| 12/01/2026 | 21:51:31 | Fault | Latch: Upper axis (Read timed out) |
| 12/01/2026 | 21:51:31 | Condition | Connectivity fault: Upper axis (Read timed out) |
| 12/01/2026 | 21:51:31 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| 12/01/2026 | 21:51:31 | Fault | Latch: Lower axis (Read timed out) |
| 12/01/2026 | 21:51:31 | Condition | Connectivity fault: Lower axis (Read timed out) |
| 12/01/2026 | 21:51:31 | Scheduler | Present item cleared |
| 12/01/2026 | 21:51:31 | Motion | Stop |
| 12/01/2026 | 21:51:31 | Scheduler | Blocked |
| 12/01/2026 | 21:51:32 | Comms | [CGC HPA]: I/O error: Read timed out |
| 12/01/2026 | 21:51:32 | Fault | Latch: HPA (Read timed out) |
| 12/01/2026 | 21:51:32 | Condition | Connectivity warning: HPA (Read timed out) |
| 12/01/2026 | 21:51:32 | Comms | [Advantech ADAM-5000]: I/O error: Read timed out |
| 12/01/2026 | 21:51:32 | Fault | Latch: Logic controller (Read timed out) |
| 12/01/2026 | 21:51:32 | Condition | Connectivity fault: Logic controller (Read timed out) |
| 12/01/2026 | 21:51:33 | Fault | Latch: Clinometer (Read timeout (X)) |
| 12/01/2026 | 21:51:33 | Condition | Connectivity fault: Clinometer (Read timeout (X)) |
| 12/01/2026 | 21:51:33 | Fault | Latch: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 21:51:33 | Condition | Connectivity warning: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 21:51:33 | Fault | Latch: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 21:51:33 | Condition | Connectivity warning: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 21:51:33 | Fault | Latch: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 21:51:33 | Condition | Connectivity warning: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 21:51:34 | Fault | Latch: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 21:51:34 | Condition | Connectivity warning: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 21:51:36 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 21:51:36 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 21:51:36 | Comms | [MicroFlex E150]: Connecting to 192.168.100.110:5000 with timeout 1000ms |
| 12/01/2026 | 21:51:36 | Comms | [MicroFlex E150]: Connecting to 192.168.100.111:5000 with timeout 1000ms |
| 12/01/2026 | 21:51:36 | Comms | [MicroFlex E150]: Fetching I/O streams |
| 12/01/2026 | 21:51:36 | Comms | [MicroFlex E150]: Connected |
| 12/01/2026 | 21:51:36 | Comms | [MicroFlex E150]: Fetching I/O streams |
| 12/01/2026 | 21:51:36 | Comms | [MicroFlex E150]: Connected |
| 12/01/2026 | 21:51:37 | Fault | Reset: Lower axis |
| 12/01/2026 | 21:51:37 | Fault | Reset: Upper axis |
| 12/01/2026 | 21:51:37 | Hardware | Starting CGC HPA |
| 12/01/2026 | 21:51:37 | Hardware | Starting Advantech ADAM-5000 |
| 12/01/2026 | 21:51:37 | Comms | [CGC HPA]: Connecting to 192.168.100.72:23 with timeout 1000ms |
| 12/01/2026 | 21:51:37 | Comms | [Advantech ADAM-5000]: Connecting to 192.168.100.68:502 with timeout 2000ms |
| 12/01/2026 | 21:51:37 | Comms | [CGC HPA]: Fetching I/O streams |
| 12/01/2026 | 21:51:37 | Comms | [CGC HPA]: Connected |
| 12/01/2026 | 21:51:37 | Comms | [Advantech ADAM-5000]: Fetching I/O streams |
| 12/01/2026 | 21:51:37 | Comms | [Advantech ADAM-5000]: Connected |
| 12/01/2026 | 21:51:37 | Fault | Reset: Logic controller |
| 12/01/2026 | 21:51:38 | Hardware | Starting Novella D688 |
| 12/01/2026 | 21:51:38 | Hardware | Starting Novella D688 |
| 12/01/2026 | 21:51:38 | Hardware | Starting Novella D492 |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D492]: Initialising to 192.168.100.53:161 |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D688]: Initialising to 192.168.100.50:161 |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D688]: Initialising to 192.168.100.51:161 |
| 12/01/2026 | 21:51:38 | Hardware | Starting Altheris NS/P2 |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D492]: Creating connection |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D492]: Connected |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D688]: Connected |
| 12/01/2026 | 21:51:38 | SNMP | [Novella D688]: Connected |
| 12/01/2026 | 21:51:38 | Fault | Reset: S-Band Downconverter |
| 12/01/2026 | 21:51:38 | Fault | Reset: X-Band RHCP Downconverter |
| 12/01/2026 | 21:51:38 | Fault | Reset: X-Band LHCP Downconverter |
| 12/01/2026 | 21:51:39 | Fault | Reset: HPA |
| 12/01/2026 | 21:51:39 | Hardware | Starting Novella U492 |
| 12/01/2026 | 21:51:39 | SNMP | [Novella U492]: Initialising to 192.168.100.52:161 |
| 12/01/2026 | 21:51:39 | SNMP | [Novella U492]: Creating connection |
| 12/01/2026 | 21:51:39 | SNMP | [Novella U492]: Connected |
| 12/01/2026 | 21:51:39 | Fault | Reset: S-Band Upconverter |
| 12/01/2026 | 21:51:40 | Fault | Reset: Clinometer |
| 12/01/2026 | 21:51:40 | Scheduler | Released |
| 12/01/2026 | 21:51:40 | Scheduler | Next item is now DIWATA-2B (2026-01-12 22:44:39, 2026-01-12 22:50:50) |
| 12/01/2026 | 22:09:35 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| 12/01/2026 | 22:09:35 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| 12/01/2026 | 22:09:35 | Fault | Latch: Lower axis (Read timed out) |
| 12/01/2026 | 22:09:35 | Fault | Latch: Upper axis (Read timed out) |
| 12/01/2026 | 22:09:35 | Condition | Connectivity fault: Upper axis (Read timed out) |
| 12/01/2026 | 22:09:35 | Condition | Connectivity fault: Lower axis (Read timed out) |
| 12/01/2026 | 22:09:35 | Scheduler | Present item cleared |
| 12/01/2026 | 22:09:35 | Motion | Stop |
| 12/01/2026 | 22:09:35 | Scheduler | Blocked |
| 12/01/2026 | 22:09:36 | Comms | [CGC HPA]: I/O error: Read timed out |
| 12/01/2026 | 22:09:36 | Fault | Latch: HPA (Read timed out) |
| 12/01/2026 | 22:09:36 | Condition | Connectivity warning: HPA (Read timed out) |
| 12/01/2026 | 22:09:36 | Comms | [Advantech ADAM-5000]: I/O error: Read timed out |
| 12/01/2026 | 22:09:36 | Fault | Latch: Logic controller (Read timed out) |
| 12/01/2026 | 22:09:36 | Condition | Connectivity fault: Logic controller (Read timed out) |
| 12/01/2026 | 22:09:37 | Fault | Latch: Clinometer (Read timeout (X)) |
| 12/01/2026 | 22:09:37 | Condition | Connectivity fault: Clinometer (Read timeout (X)) |
| 12/01/2026 | 22:09:38 | Fault | Latch: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 22:09:38 | Condition | Connectivity warning: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 22:09:38 | Fault | Latch: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 22:09:38 | Condition | Connectivity warning: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 22:09:38 | Fault | Latch: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 22:09:38 | Fault | Latch: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 22:09:38 | Condition | Connectivity warning: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 22:09:38 | Condition | Connectivity warning: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 22:09:40 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 22:09:40 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 22:09:40 | Comms | [MicroFlex E150]: Connecting to 192.168.100.110:5000 with timeout 1000ms |
| 12/01/2026 | 22:09:40 | Comms | [MicroFlex E150]: Connecting to 192.168.100.111:5000 with timeout 1000ms |
| 12/01/2026 | 22:09:41 | Hardware | Starting Advantech ADAM-5000 |
| 12/01/2026 | 22:09:41 | Hardware | Starting CGC HPA |
| 12/01/2026 | 22:09:41 | Comms | [Advantech ADAM-5000]: Connecting to 192.168.100.68:502 with timeout 2000ms |
| 12/01/2026 | 22:09:41 | Comms | [CGC HPA]: Connecting to 192.168.100.72:23 with timeout 1000ms |
| 12/01/2026 | 22:09:41 | Comms | [MicroFlex E150]: Failed to connect: Connect timed out |
| 12/01/2026 | 22:09:41 | Comms | [MicroFlex E150]: Failed to connect: Connect timed out |
| 12/01/2026 | 22:09:41 | Fault | Latch: Lower axis (Connect timed out) |
| 12/01/2026 | 22:09:41 | Fault | Latch: Upper axis (Connect timed out) |
| 12/01/2026 | 22:09:41 | Condition | Connectivity fault: Lower axis (Connect timed out) |
| 12/01/2026 | 22:09:41 | Condition | Connectivity fault: Upper axis (Connect timed out) |
| 12/01/2026 | 22:09:42 | Hardware | Starting Altheris NS/P2 |
| 12/01/2026 | 22:09:42 | Comms | [CGC HPA]: Failed to connect: Connect timed out |
| 12/01/2026 | 22:09:42 | Fault | Latch: HPA (Connect timed out) |
| 12/01/2026 | 22:09:42 | Condition | Connectivity warning: HPA (Connect timed out) |
| 12/01/2026 | 22:09:43 | Comms | [Advantech ADAM-5000]: Failed to connect: Connect timed out |
| 12/01/2026 | 22:09:43 | Fault | Latch: Logic controller (Connect timed out) |
| 12/01/2026 | 22:09:43 | Condition | Connectivity fault: Logic controller (Connect timed out) |
| 12/01/2026 | 22:09:44 | Hardware | Starting Novella D688 |
| 12/01/2026 | 22:09:44 | Hardware | Starting Novella U492 |
| 12/01/2026 | 22:09:44 | Hardware | Starting Novella D492 |
| 12/01/2026 | 22:09:44 | SNMP | [Novella D492]: Initialising to 192.168.100.53:161 |
| 12/01/2026 | 22:09:44 | SNMP | [Novella D688]: Initialising to 192.168.100.51:161 |
| 12/01/2026 | 22:09:44 | Hardware | Starting Novella D688 |
| 12/01/2026 | 22:09:44 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 22:09:44 | SNMP | [Novella D492]: Creating connection |
| 12/01/2026 | 22:09:44 | SNMP | [Novella U492]: Initialising to 192.168.100.52:161 |
| 12/01/2026 | 22:09:44 | SNMP | [Novella D688]: Initialising to 192.168.100.50:161 |
| 12/01/2026 | 22:09:44 | SNMP | [Novella U492]: Creating connection |
| 12/01/2026 | 22:09:44 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 22:09:45 | Fault | Latch: Clinometer (Connect timed out) |
| 12/01/2026 | 22:09:45 | Condition | Connectivity fault: Clinometer (Connect timed out) |
| 12/01/2026 | 22:09:46 | SNMP | [Novella D688]: Failed to connect: No response |
| 12/01/2026 | 22:09:46 | SNMP | [Novella U492]: Failed to connect: No response |
| 12/01/2026 | 22:09:46 | SNMP | [Novella D688]: Failed to connect: No response |
| 12/01/2026 | 22:09:46 | SNMP | [Novella D492]: Failed to connect: No response |
| 12/01/2026 | 22:09:46 | Fault | Latch: X-Band RHCP Downconverter (No response) |
| 12/01/2026 | 22:09:46 | Condition | Connectivity warning: X-Band RHCP Downconverter (No response) |
| 12/01/2026 | 22:09:46 | Fault | Latch: S-Band Upconverter (No response) |
| 12/01/2026 | 22:09:46 | Condition | Connectivity warning: S-Band Upconverter (No response) |
| 12/01/2026 | 22:09:46 | Fault | Latch: X-Band LHCP Downconverter (No response) |
| 12/01/2026 | 22:09:46 | Fault | Latch: S-Band Downconverter (No response) |
| 12/01/2026 | 22:09:46 | Condition | Connectivity warning: S-Band Downconverter (No response) |
| 12/01/2026 | 22:09:46 | Condition | Connectivity warning: X-Band LHCP Downconverter (No response) |
| 12/01/2026 | 22:09:51 | Fault | Reset: S-Band Downconverter |
| 12/01/2026 | 22:09:51 | Fault | Reset: X-Band LHCP Downconverter |
| 12/01/2026 | 22:09:51 | Fault | Reset: X-Band RHCP Downconverter |
| 12/01/2026 | 22:09:51 | Fault | Reset: S-Band Upconverter |
| 12/01/2026 | 22:09:53 | Fault | Reset: Upper axis |
| 12/01/2026 | 22:09:53 | Fault | Reset: Lower axis |
| 12/01/2026 | 22:09:53 | Fault | Latch: Clinometer (Unable to read clinometer values) |
| 12/01/2026 | 22:09:53 | Condition | Connectivity fault: Clinometer (Unable to read clinometer values) |
| 12/01/2026 | 22:09:53 | Fault | Reset: Clinometer |
| 12/01/2026 | 22:09:54 | Fault | Latch: Clinometer (Socket closed) |
| 12/01/2026 | 22:09:54 | Condition | Connectivity fault: Clinometer (Socket closed) |
| 12/01/2026 | 22:09:54 | Fault | Reset: HPA |
| 12/01/2026 | 22:09:55 | Fault | Reset: Logic controller |
| 12/01/2026 | 22:09:59 | Hardware | Starting Altheris NS/P2 |
| 12/01/2026 | 22:10:00 | Fault | Reset: Clinometer |
| 12/01/2026 | 22:10:01 | Scheduler | Released |
| 12/01/2026 | 22:10:01 | Scheduler | Next item is now DIWATA-2B (2026-01-12 22:44:39, 2026-01-12 22:50:50) |
| 12/01/2026 | 22:39:39 | IO | Reading: /home/cgc/Desktop/TLE/AIM |
| 12/01/2026 | 22:39:39 | IO | Error (/home/cgc/Desktop/TLE/AIM (No such file or directory)): /home/cgc/Desktop/TLE/AIM |
| 12/01/2026 | 22:39:39 | IO | Reading: /home/acu/Desktop/TLE/./tleupdate_log.txt |
| 12/01/2026 | 22:39:39 | IO | Closing: /home/acu/Desktop/TLE/./tleupdate_log.txt |
| 12/01/2026 | 22:39:39 | IO | Reading: /home/acu/Desktop/TLE/./2025_TLE.txt |
| 12/01/2026 | 22:39:39 | IO | Closing: /home/acu/Desktop/TLE/./2025_TLE.txt |
| 12/01/2026 | 22:44:00 | Motion | Releasing brake on east axis |
| 12/01/2026 | 22:44:00 | Motion | Releasing brake on north axis |
| 12/01/2026 | 22:44:00 | Track | Intercepting DIWATA-2B (2026-01-12 22:44:39, 2026-01-12 22:50:50) |
| 12/01/2026 | 22:44:00 | Scheduler | Launching DIWATA-2B (2026-01-12 22:44:39, 2026-01-12 22:50:50) |
| 12/01/2026 | 22:44:00 | Motion | Go |
| 12/01/2026 | 22:44:23 | Motion | Idle |
| 12/01/2026 | 22:44:25 | Motion | Applying brake on east axis |
| 12/01/2026 | 22:44:25 | Motion | Applying brake on north axis |
| 12/01/2026 | 22:44:35 | Motion | Releasing brake on east axis |
| 12/01/2026 | 22:44:35 | Motion | Releasing brake on north axis |
| 12/01/2026 | 22:44:40 | Track | Launching DIWATA-2B (2026-01-12 22:44:39, 2026-01-12 22:50:50) |
| 12/01/2026 | 22:44:40 | Action | Running macro: Diwata1 |
| 12/01/2026 | 22:44:40 | Action | Starting event actions: Diwata1 |
| 12/01/2026 | 22:44:40 | Action | Running macro: diwata1 uplink |
| 12/01/2026 | 22:44:40 | Action | Starting event actions: diwata1 uplink |
| 12/01/2026 | 22:44:40 | Motion | Go |
| 12/01/2026 | 22:50:51 | Motion | Stop |
| 12/01/2026 | 22:50:51 | Scheduler | Present item cleared |
| 12/01/2026 | 22:50:51 | Motion | Stop |
| 12/01/2026 | 22:50:51 | Scheduler | Stopping event (Trackable path depleted) |
| 12/01/2026 | 22:50:51 | Motion | Idle |
| 12/01/2026 | 22:50:51 | Scheduler | Removing expired event: DIWATA-2B (2026-01-12 22:44:39, 2026-01-12 22:50:50) |
| 12/01/2026 | 22:50:51 | Scheduler | Next item is now NOVASAR 1 (2026-01-13 00:49:23, 2026-01-13 00:59:02) |
| 12/01/2026 | 22:50:51 | Scheduler | Stowing antenna |
| 12/01/2026 | 22:50:51 | Motion | Go |
| 12/01/2026 | 22:51:14 | Motion | Idle |
| 12/01/2026 | 22:51:16 | Motion | Applying brake on east axis |
| 12/01/2026 | 22:51:16 | Motion | Applying brake on north axis |
| 12/01/2026 | 23:37:42 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| 12/01/2026 | 23:37:42 | Fault | Latch: Upper axis (Read timed out) |
| 12/01/2026 | 23:37:42 | Condition | Connectivity fault: Upper axis (Read timed out) |
| 12/01/2026 | 23:37:42 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| 12/01/2026 | 23:37:42 | Fault | Latch: Lower axis (Read timed out) |
| 12/01/2026 | 23:37:42 | Condition | Connectivity fault: Lower axis (Read timed out) |
| 12/01/2026 | 23:37:42 | Scheduler | Present item cleared |
| 12/01/2026 | 23:37:42 | Motion | Stop |
| 12/01/2026 | 23:37:42 | Scheduler | Blocked |
| 12/01/2026 | 23:37:42 | Comms | [CGC HPA]: I/O error: Read timed out |
| 12/01/2026 | 23:37:42 | Fault | Latch: HPA (Read timed out) |
| 12/01/2026 | 23:37:42 | Condition | Connectivity warning: HPA (Read timed out) |
| 12/01/2026 | 23:37:43 | Comms | [Advantech ADAM-5000]: I/O error: Read timed out |
| 12/01/2026 | 23:37:43 | Fault | Latch: Logic controller (Read timed out) |
| 12/01/2026 | 23:37:43 | Condition | Connectivity fault: Logic controller (Read timed out) |
| 12/01/2026 | 23:37:44 | Fault | Latch: Clinometer (Read timeout (X)) |
| 12/01/2026 | 23:37:44 | Condition | Connectivity fault: Clinometer (Read timeout (X)) |
| 12/01/2026 | 23:37:44 | Fault | Latch: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 23:37:44 | Condition | Connectivity warning: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 23:37:44 | Fault | Latch: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:37:44 | Condition | Connectivity warning: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:37:44 | Fault | Latch: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:37:44 | Condition | Connectivity warning: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:37:44 | Fault | Latch: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 23:37:44 | Condition | Connectivity warning: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 23:37:48 | Hardware | Starting CGC HPA |
| 12/01/2026 | 23:37:48 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 23:37:48 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 23:37:48 | Comms | [MicroFlex E150]: Connecting to 192.168.100.111:5000 with timeout 1000ms |
| 12/01/2026 | 23:37:48 | Comms | [CGC HPA]: Connecting to 192.168.100.72:23 with timeout 1000ms |
| 12/01/2026 | 23:37:48 | Comms | [MicroFlex E150]: Connecting to 192.168.100.110:5000 with timeout 1000ms |
| 12/01/2026 | 23:37:48 | Comms | [CGC HPA]: Fetching I/O streams |
| 12/01/2026 | 23:37:48 | Comms | [CGC HPA]: Connected |
| 12/01/2026 | 23:37:48 | Comms | [MicroFlex E150]: Fetching I/O streams |
| 12/01/2026 | 23:37:48 | Comms | [MicroFlex E150]: Connected |
| 12/01/2026 | 23:37:48 | Comms | [MicroFlex E150]: Fetching I/O streams |
| 12/01/2026 | 23:37:48 | Comms | [MicroFlex E150]: Connected |
| 12/01/2026 | 23:37:48 | Fault | Reset: Lower axis |
| 12/01/2026 | 23:37:48 | Fault | Reset: Upper axis |
| 12/01/2026 | 23:37:48 | Hardware | Starting Advantech ADAM-5000 |
| 12/01/2026 | 23:37:48 | Comms | [Advantech ADAM-5000]: Connecting to 192.168.100.68:502 with timeout 2000ms |
| 12/01/2026 | 23:37:48 | Comms | [Advantech ADAM-5000]: Fetching I/O streams |
| 12/01/2026 | 23:37:48 | Comms | [Advantech ADAM-5000]: Connected |
| 12/01/2026 | 23:37:48 | Fault | Reset: Logic controller |
| 12/01/2026 | 23:37:49 | Fault | Reset: HPA |
| 12/01/2026 | 23:37:49 | Hardware | Starting Novella D492 |
| 12/01/2026 | 23:37:49 | Hardware | Starting Altheris NS/P2 |
| 12/01/2026 | 23:37:49 | Hardware | Starting Novella U492 |
| 12/01/2026 | 23:37:49 | SNMP | [Novella U492]: Initialising to 192.168.100.52:161 |
| 12/01/2026 | 23:37:49 | Hardware | Starting Novella D688 |
| 12/01/2026 | 23:37:49 | SNMP | [Novella U492]: Creating connection |
| 12/01/2026 | 23:37:49 | Hardware | Starting Novella D688 |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D688]: Initialising to 192.168.100.50:161 |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D492]: Initialising to 192.168.100.53:161 |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D492]: Creating connection |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D688]: Initialising to 192.168.100.51:161 |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D492]: Connected |
| 12/01/2026 | 23:37:49 | SNMP | [Novella U492]: Connected |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D688]: Connected |
| 12/01/2026 | 23:37:49 | SNMP | [Novella D688]: Connected |
| 12/01/2026 | 23:37:49 | Fault | Reset: S-Band Downconverter |
| 12/01/2026 | 23:37:49 | Fault | Reset: S-Band Upconverter |
| 12/01/2026 | 23:37:49 | Fault | Reset: X-Band RHCP Downconverter |
| 12/01/2026 | 23:37:49 | Fault | Reset: X-Band LHCP Downconverter |
| 12/01/2026 | 23:37:51 | Fault | Reset: Clinometer |
| 12/01/2026 | 23:37:51 | Scheduler | Released |
| 12/01/2026 | 23:37:51 | Scheduler | Next item is now NOVASAR 1 (2026-01-13 00:49:23, 2026-01-13 00:59:02) |
| 12/01/2026 | 23:39:39 | IO | Reading: /home/cgc/Desktop/TLE/AIM |
| 12/01/2026 | 23:39:39 | IO | Error (/home/cgc/Desktop/TLE/AIM (No such file or directory)): /home/cgc/Desktop/TLE/AIM |
| 12/01/2026 | 23:39:39 | IO | Reading: /home/acu/Desktop/TLE/./tleupdate_log.txt |
| 12/01/2026 | 23:39:39 | IO | Closing: /home/acu/Desktop/TLE/./tleupdate_log.txt |
| 12/01/2026 | 23:39:39 | IO | Reading: /home/acu/Desktop/TLE/./2025_TLE.txt |
| 12/01/2026 | 23:39:39 | IO | Closing: /home/acu/Desktop/TLE/./2025_TLE.txt |
| 12/01/2026 | 23:41:06 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| 12/01/2026 | 23:41:06 | Comms | [MicroFlex E150]: I/O error: Read timed out |
| 12/01/2026 | 23:41:06 | Fault | Latch: Lower axis (Read timed out) |
| 12/01/2026 | 23:41:06 | Fault | Latch: Upper axis (Read timed out) |
| 12/01/2026 | 23:41:06 | Condition | Connectivity fault: Upper axis (Read timed out) |
| 12/01/2026 | 23:41:06 | Scheduler | Present item cleared |
| 12/01/2026 | 23:41:06 | Motion | Stop |
| 12/01/2026 | 23:41:06 | Scheduler | Blocked |
| 12/01/2026 | 23:41:06 | Condition | Connectivity fault: Lower axis (Read timed out) |
| 12/01/2026 | 23:41:07 | Comms | [CGC HPA]: I/O error: Read timed out |
| 12/01/2026 | 23:41:07 | Fault | Latch: HPA (Read timed out) |
| 12/01/2026 | 23:41:07 | Condition | Connectivity warning: HPA (Read timed out) |
| 12/01/2026 | 23:41:07 | Comms | [Advantech ADAM-5000]: I/O error: Read timed out |
| 12/01/2026 | 23:41:07 | Fault | Latch: Logic controller (Read timed out) |
| 12/01/2026 | 23:41:07 | Condition | Connectivity fault: Logic controller (Read timed out) |
| 12/01/2026 | 23:41:08 | Fault | Latch: Clinometer (Read timeout (X)) |
| 12/01/2026 | 23:41:08 | Condition | Connectivity fault: Clinometer (Read timeout (X)) |
| 12/01/2026 | 23:41:09 | Fault | Latch: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:41:09 | Fault | Latch: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:41:09 | Condition | Connectivity warning: X-Band RHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:41:09 | Condition | Connectivity warning: X-Band LHCP Downconverter (Request failed: No response) |
| 12/01/2026 | 23:41:09 | Fault | Latch: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 23:41:09 | Condition | Connectivity warning: S-Band Upconverter (Request failed: No response) |
| 12/01/2026 | 23:41:09 | Fault | Latch: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 23:41:09 | Condition | Connectivity warning: S-Band Downconverter (Request failed: No response) |
| 12/01/2026 | 23:41:11 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 23:41:11 | Hardware | Starting MicroFlex E150 |
| 12/01/2026 | 23:41:11 | Comms | [MicroFlex E150]: Connecting to 192.168.100.110:5000 with timeout 1000ms |
| 12/01/2026 | 23:41:11 | Comms | [MicroFlex E150]: Connecting to 192.168.100.111:5000 with timeout 1000ms |
| 12/01/2026 | 23:41:12 | Comms | [MicroFlex E150]: Failed to connect: Connect timed out |
| 12/01/2026 | 23:41:12 | Fault | Latch: Upper axis (Connect timed out) |
| 12/01/2026 | 23:41:12 | Condition | Connectivity fault: Upper axis (Connect timed out) |
| 12/01/2026 | 23:41:12 | Comms | [MicroFlex E150]: Failed to connect: Connect timed out |
| 12/01/2026 | 23:41:12 | Fault | Latch: Lower axis (Connect timed out) |
| 12/01/2026 | 23:41:12 | Condition | Connectivity fault: Lower axis (Connect timed out) |
| 12/01/2026 | 23:41:12 | Hardware | Starting Advantech ADAM-5000 |
| 12/01/2026 | 23:41:12 | Hardware | Starting CGC HPA |
| 12/01/2026 | 23:41:12 | Comms | [Advantech ADAM-5000]: Connecting to 192.168.100.68:502 with timeout 2000ms |
| 12/01/2026 | 23:41:12 | Comms | [CGC HPA]: Connecting to 192.168.100.72:23 with timeout 1000ms |
| 12/01/2026 | 23:41:13 | Hardware | Starting Altheris NS/P2 |
| 12/01/2026 | 23:41:13 | Comms | [CGC HPA]: Failed to connect: Connect timed out |
| 12/01/2026 | 23:41:13 | Fault | Latch: HPA (Connect timed out) |
| 12/01/2026 | 23:41:13 | Condition | Connectivity warning: HPA (Connect timed out) |
| 12/01/2026 | 23:41:14 | Hardware | Starting Novella D688 |
| 12/01/2026 | 23:41:14 | Hardware | Starting Novella U492 |
| 12/01/2026 | 23:41:14 | Hardware | Starting Novella D492 |
| 12/01/2026 | 23:41:14 | Hardware | Starting Novella D688 |
| 12/01/2026 | 23:41:14 | SNMP | [Novella D688]: Initialising to 192.168.100.50:161 |
| 12/01/2026 | 23:41:14 | SNMP | [Novella D688]: Initialising to 192.168.100.51:161 |
| 12/01/2026 | 23:41:14 | SNMP | [Novella D492]: Initialising to 192.168.100.53:161 |
| 12/01/2026 | 23:41:14 | SNMP | [Novella U492]: Initialising to 192.168.100.52:161 |
| 12/01/2026 | 23:41:14 | SNMP | [Novella D492]: Creating connection |
| 12/01/2026 | 23:41:14 | SNMP | [Novella U492]: Creating connection |
| 12/01/2026 | 23:41:14 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 23:41:14 | SNMP | [Novella D688]: Creating connection |
| 12/01/2026 | 23:41:14 | Comms | [Advantech ADAM-5000]: Failed to connect: Connect timed out |
| 12/01/2026 | 23:41:14 | Fault | Latch: Logic controller (Connect timed out) |
| 12/01/2026 | 23:41:14 | Condition | Connectivity fault: Logic controller (Connect timed out) |
| 12/01/2026 | 23:41:16 | Fault | Latch: Clinometer (Connect timed out) |
| 12/01/2026 | 23:41:16 | Condition | Connectivity fault: Clinometer (Connect timed out) |
| 12/01/2026 | 23:41:16 | SNMP | [Novella U492]: Connected |
| 12/01/2026 | 23:41:16 | SNMP | [Novella D688]: Connected |
| 12/01/2026 | 23:41:16 | SNMP | [Novella D492]: Connected |
| 12/01/2026 | 23:41:16 | SNMP | [Novella D688]: Connected |
| 12/01/2026 | 23:41:16 | Fault | Reset: S-Band Upconverter |
| 12/01/2026 | 23:41:16 | Fault | Reset: S-Band Downconverter |
| 12/01/2026 | 23:41:16 | Fault | Reset: X-Band LHCP Downconverter |
| 12/01/2026 | 23:41:16 | Fault | Reset: X-Band RHCP Downconverter |
| 12/01/2026 | 23:41:18 | Fault | Reset: Upper axis |
| 12/01/2026 | 23:41:18 | Fault | Reset: Lower axis |
| 12/01/2026 | 23:41:20 | Fault | Reset: Logic controller |
| 12/01/2026 | 23:41:21 | Fault | Reset: HPA |
| 12/01/2026 | 23:41:22 | Fault | Reset: Clinometer |
| 12/01/2026 | 23:41:22 | Scheduler | Released |
| 12/01/2026 | 23:41:22 | Scheduler | Next item is now NOVASAR 1 (2026-01-13 00:49:23, 2026-01-13 00:59:02) |

## Damages Incurred (if any)

None

# Response and Actions Taken

| Date | Time | Performed by | Action | Result |
| --- | --- | --- | --- | --- |
| 2026-01-13 | 9:00 | Kez | Review Logs | - |

# Investigation and Analysis

There were no reports of power interruptions at DOST-ASTI where the equipment is hosted. Hence, it is likely that the fault is in the network switch found at the RF cabinet.

# Conclusion and Recommendations

Market study for a new network switch should be prepared in case this issue persists. Replacement of the network switch is recommended.